The preferred minimum roadway section for minor residential streets is as follows:

Scarify, moisture treat and compact subgrade to 95% max. theoretical density.

Place 12” flex base in 6” lifts compacted to 100% max. theoretical density.

Prime coat.

1-1/2” Type D HMAC

14 foot minimum width